HOLY CROSS COLLEGE

Sta. Lucia, Sta. Ana, Pampanga

School of Engineering, Computer and Library Studies

Firs Semester, A.Y. 2024-2025

Midterm Examination in Fundamentals of IoT 1

Name:	______________________________________	Score:_________________________

Course-Year and Section: ______________________	Date  :________________________

Midterm Exam Instructions: Home Automation System

- Overview:

You will work in groups of up to 5 members to create a Home Automation System using Wokwi Simulation. The system must integrate multiple sensors and actuators to simulate a smart environment. The components and their respective behaviors are detailed below.

- System Requirements:

- DHT22 Sensor (Temperature & Humidity):

  - The temperature and humidity readings should be displayed on an I2C LCD.

  - LED indicators should respond based on the temperature:

    - Green LED: Turns on when the temperature is below 25°C.

    - Yellow LED: Turns on when the temperature is between 25°C and 35°C.

    - Red LED: Turns on when the temperature exceeds 35°C.

- Light Sensor:

  - The system must monitor the light intensity (lux).

  - When the light level exceeds 500 lux, the servo motor should rotate to 360 degrees.

  - When the light level drops below 500 lux, the servo motor should return to 0 degrees.

- Motion Sensor (PIR):

  - When motion is detected, a buzzer should sound for 3 seconds.

- Tools and Simulation:

- You are required to use Wokwi Simulation to design, code, and test your system.

- All connections, components, and coding must be properly simulated.

- Group Formation:

- Form groups of up to 6 members.

Grading Criteria (Total: 100 points):

- System Functionality (60 points): Proper operation of all components as described.

- Creativity and Innovation (20 points): Any additional features or design improvements.

- Code Efficiency and Clarity (20 points): Well-written, commented code that follows programming standards.

| Prepared by: Engr. Clarence R. Estacio Instructor | Checked by: Romalyn H. Gomez, MEnM, ECE Program Chair, CS/ACT | Checked by: Arsel D. Pinpin, MIT Program Chair, IT |
| --- | --- | --- |
| Noted by: Marvin O. Mallari, PCpE, Ph. D. Dean, SECLS | Approved by: Leticia D. Flores, Ph.D Vice President for Academic Affairs |  |